By:  LucioS.B. No. 623

A BILL TO BE ENTITLED

AN ACT

relating to certain requirements applicable to the construction of affordable housing.

BE IT ENACTED BY THE LEGISLATURE OF THE STATE OF TEXAS:

SECTION 1.  Subchapter X, Chapter 2306, Government Code, is amended by adding Section 2306.514 to read as follows:

Sec. 2306.514.  CONSTRUCTION REQUIREMENTS FOR AFFORDABLE HOUSING.  (a)  If a person is awarded state or federal funds by the department to construct affordable housing for individuals and families of low and very low income, the affordable housing identified on the person's funding application must be constructed so that:

(1)  at least one entrance door, whether located at the front, side, or back of the building:

(A)  is on an accessible route served by a ramp or no-step entrance; and

(B)  has a net clear opening of at least 30 inches;

(2)  on the first floor of the building:

(A)  each interior door is a standard 30-inch door, unless the door provides access only to a closet of less than 15 square feet in area;

(B)  each hallway has a width of at least 36 inches and is level, with ramped or beveled changes at each door threshold;

(C)  each bathroom wall is reinforced for potential installation of grab bars;

(D)  each electrical panel or breaker box, light switch, or thermostat is not higher than 48 inches above the floor; and

(E)  each electrical plug or other receptacle is at least 15 inches above the floor; and

(3)  each breaker box is located inside the building on the first floor.

(b)  A person who builds affordable housing to which this section applies may obtain a waiver from the department of the requirement described by Subsection (a)(1)(A) if the cost of grading the terrain to meet the requirement is prohibitively expensive.

SECTION 2.  The change in law made by this Act applies only to affordable housing for which new construction begins on or after the effective date of this Act.  Affordable housing for which new construction begins before the effective date of this Act is covered by the law in effect when the construction began, and the former law is continued in effect for that purpose.

SECTION 3.  This Act takes effect September 1, 1999.

SECTION 4.  The importance of this legislation and the crowded condition of the calendars in both houses create an emergency and an imperative public necessity that the constitutional rule requiring bills to be read on three several days in each house be suspended, and this rule is hereby suspended.

COMMITTEE AMENDMENT NO. 1

Amend S.B. No. 623 (senate engrossment) in SECTION 1 of the bill as follows:

(1)  In proposed Section 2306.514(a)(1)(B), Government Code (page 1, line 16), strike "30" and substitute "34".

(2)  In proposed Section 2306.514(a)(2)(A), Government Code (page 1, line 19), strike "a standard 30-inch" and substitute "at least a 32-inch".

Burnam