79R7128 JTS-D

By:  Puente	H.B. No. 2429

A BILL TO BE ENTITLED

AN ACT

relating to water and energy saving performance standards for commercial clothes-washing machines.

BE IT ENACTED BY THE LEGISLATURE OF THE STATE OF TEXAS:

SECTION 1.  Section 372.001, Health and Safety Code, is amended by adding a new Subdivision (1) and renumbering existing Subdivision (1) as Subdivision (1-a) and amending that subdivision to read as follows:

(1)  "Commercial clothes-washing machine" means a clothes washer that:

(A)  is designed to be used in a commercial setting, including multifamily housing, an apartment, or a coin-operated laundry; and

(B)  has a capacity of:

(i)  not more than 3.5 cubic feet if it is a horizontal axis machine; or

(ii)  not more than 4 cubic feet if it is a vertical axis machine.

(1-a)  "Commission" means the Texas [Natural Resource Conservation] Commission on Environmental Quality.

SECTION 2.  Chapter 372, Health and Safety Code, is amended by adding Section 372.005 to read as follows:

Sec. 372.005.  COMMERCIAL CLOTHES-WASHING MACHINE PERFORMANCE STANDARDS.  (a) A person may not sell, offer for sale, distribute, lease, or import into this state a commercial clothes-washing machine for use in this state unless the machine:

(1)  meets the water and energy saving performance standards provided by Subsection (b); and

(2)  is listed by the commission under Subsection (c).

(b)  For a commercial clothes-washing machine, the water consumption factor may not exceed 9.5 gallons per cubic foot per cycle and the modified energy factor must be at least 1.26 cubic feet per kilowatt-hours per cycle, as determined using the test procedures established by the United States Department of Energy as specified in 10 C.F.R. Part 430, Subpart B, Appendix J1, as that appendix existed on October 31, 2003.

(c)  The commission shall make and maintain a current list of commercial clothes-washing machines that are certified to the commission by the manufacturer or importer to meet the water and energy saving performance standards provided by Subsection (b).  To have a commercial clothes-washing machine included on the list, the manufacturer or importer must supply to the commission, in the form prescribed by the commission, the identification and the performance specifications of the commercial clothes-washing machine.  The commission may test a listed commercial clothes-washing machine to determine the accuracy of the manufacturer's or importer's certification and shall remove from the list a machine the commission determines to be inaccurately certified.

(d)  The commission may include on the list described by Subsection (c) commercial clothes-washing machines that are certified by the Consortium for Energy Efficiency, Inc., to meet the  water and energy saving performance standards provided by Subsection (b).

(e)  The commission may assess against a manufacturer or an importer a reasonable fee for an inspection of a commercial clothes-washing machine to determine the accuracy of the manufacturer's or importer's certification in an amount determined by the commission to cover the expenses incurred in the administration of this chapter.  A fee received by the commission under this subsection shall be deposited to the credit of the water resource management account in the general revenue fund. Fees deposited under this section may be appropriated only for the administration of this chapter.

(f)  The commission shall, to the extent appropriate and practical, employ the standards established by the United States Department of Energy in determining or evaluating performance standards or testing procedures under this chapter.

SECTION 3.  Section 372.005, Health and Safety Code, as added by this Act, does not apply to a commercial clothes-washing machine that as of January 1, 2007:

(1)  is in the inventory of a commercial clothes-washing machine retailer, distributor, lessor, or importer; or

(2)  has been ordered by a commercial clothes-washing machine retailer, distributor, lessor, or importer for delivery on or before January 31, 2007.

SECTION 4.  This Act takes effect January 1, 2007.