By:  Phillips	H.B. No. 3419

A BILL TO BE ENTITLED

AN ACT

relating to certain concrete plants issued a standard permit by the Texas Commission on Environmental Quality; amending a provision subject to a criminal penalty.

BE IT ENACTED BY THE LEGISLATURE OF THE STATE OF TEXAS:

SECTION 1.  Section 382.05198(a), Health and Safety Code, is amended to read as follows:

(a)  The commission shall issue a standard permit for a permanent concrete plant that performs wet batching, dry batching, or central mixing and that meets the following requirements:

(1)  production records must be maintained on site while the plant is in operation until the second anniversary of the end of the period to which they relate;

(2)  each cement or fly ash storage silo and weigh hopper must be equipped with a fabric or cartridge filter or vented to a fabric or cartridge filter system;

(3)  each fabric or cartridge filter, fabric or cartridge filter system, and suction shroud must be maintained and operated properly with no tears or leaks;

(4)  excluding the suction shroud filter system, each filter system must be designed to meet a standard of at least 0.01 outlet grain loading as measured in grains per dry standard cubic foot;

(5)  each filter system and each mixer loading and batch truck loading emissions control device must meet a performance standard of no visible emissions exceeding 30 seconds in a five-minute period as determined using United States Environmental Protection Agency Test Method 22 as that method existed on September 1, 2003;

(6)  if a cement or fly ash silo is filled during nondaylight hours, the silo filter system exhaust must be sufficiently illuminated to enable a determination of compliance with the performance standard described by Subdivision (5);

(7)  the conveying system for the transfer of cement or fly ash to and from each storage silo must be totally enclosed, operate properly, and be maintained without any tears or leaks;

(8)  except during cement or fly ash tanker connection or disconnection, each conveying system for the transfer of cement or fly ash must meet the performance standard described by Subdivision (5);

(9)  a warning device must be installed on each bulk storage silo to alert the operator in sufficient time for the operator to stop loading operations before the silo is filled to a level that may adversely affect the pollution abatement equipment;

(10)  if filling a silo results in failure of the pollution abatement system or failure to meet the performance standard described by Subdivision (5), the failure must be documented and reported to the commission;

(11)  each road, parking lot, or other area at the plant site that is used by vehicles must be paved with a cohesive hard surface that is properly maintained, cleaned, and watered so as to minimize dust emissions;

(12)  each stockpile must be sprinkled with water or dust-suppressant chemicals or covered so as to minimize dust emissions;

(13)  material used in the batch that is spilled must be immediately cleaned up and contained or dampened so as to minimize dust emissions;

(14)  production of concrete at the plant must not exceed 300 cubic yards per hour;

(15)  a suction shroud or other pickup device must be installed at the batch drop point or, in the case of a central mix plant, at the drum feed and vented to a fabric or cartridge filter system with a minimum capacity of 5,000 cubic feet per minute of air;

(16)  the bag filter and capture system must be properly designed to accommodate the increased flow from the suction shroud and achieve a control efficiency of at least 99.5 percent;

(17)  the suction shroud baghouse exhaust must be located more than 100 feet from any property line;

(18)  stationary equipment, stockpiles, and vehicles used at the plant, except for incidental traffic and vehicles as they enter and exit the site, must be located or operated more than 100 feet from any property line; and

(19)  the central baghouse must be located at least three miles from any building used as a school, day-care facility, or nursing home, and 1000 [440] yards from any building used as a single or multifamily residence[, school,] or place of worship at the time the application to use the permit is filed with the commission if the plant is located in an area that is not subject to municipal zoning regulation.

SECTION 2.  Section 382.05198(a), Health and Safety Code, as amended by this Act, applies only to a permit issued on or after the effective date of this Act. A permit issued before the effective date of this Act is governed by the law in effect on the date the permit is issued, and the former law is continued in effect for that purpose.

SECTION 3.  This Act takes effect September 1, 2015.